| Address | Register Name | Width | Access Type | Reset Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | CONTROL | 8 | Write-only (WO) | 0x00 | Control register for FIFO operations |
| 0x04 | STATUS | 8 | Read-only (RO) | - | Current status of the FIFO for real time monitoring. Combination of full, empty etc |
| 0x08 | ALMOST_FULL_LVL | 8 | Read/Write (RW) | 0x0C (12) | Almost Full threshold level (4 bits used) |
| 0x0C | ALMOST_EMPTY_LVL | 8 | Read/Write (RW) | 0x04 (4) | Almost Empty threshold level (4 bits used) |
| 0x10 | FIFO_DATA | 8 | Read-only (RO) | - | Peek current data (non-destructive read) |
| 0x14 | FIFO_COUNT | 8 | Read-only (RO) | - | Current number of entries in FIFO (0 to 16) |

### Detailed Breakdown of Each Register

- CONTROL Register (Address: 0x00)

  - Access: Write-only

  - Bits:

    - [1] : Clear FIFO (self-clearing pulse) — Writing 1 clears the FIFO

    - Other bits are reserved (currently not used)

  - Purpose: To issue control commands like clearing the FIFO

- STATUS Register (Address: 0x04)

  - Access: Read-only

  - Bits:

    - [0] : full

    - [1] : empty

    - [2] : almost_full

    - [3] : almost_empty

    - [4] : overflow (level sensitive)

    - [5] : underflow (level sensitive)

    - [7:6] : Reserved

  - Purpose: Real-time status monitoring

- ALMOST_FULL_LVL Register (Address: 0x08)

  - Access: Read/Write

  - Bits used: [3:0] — Threshold value (recommended range: 1 to 15)

  - Reset Value: 12

  - Purpose: Sets the level at which almost_full flag becomes active

- ALMOST_EMPTY_LVL Register (Address: 0x0C)

  - Access: Read/Write

  - Bits used: [3:0] — Threshold value

  - Reset Value: 4

  - Purpose: Sets the level at which almost_empty flag becomes active

- FIFO_DATA Register (Address: 0x10)

  - Access: Read-only (Peek)

  - Returns current head of the FIFO without popping it

  - Returns 0x00 if FIFO is empty

- FIFO_COUNT Register (Address: 0x14)

  - Access: Read-only

  - Returns current fill level (0 to 16)

  - Value is stable (registered internally)

### Important Notes:

- All registers are only accessible when cs = 1 and reg_sel = 1

- When reg_sel = 0, wr_enb and rd_enb control the actual FIFO push and pop operations.

- data_out is only updated during FIFO pop operations (reg_sel=0 and rd_enb=1)

- reg_rdata is used to return data during register reads